Network Troub.leshooting

Theme: Small Startup Office Network 

Section A: Scenario-Based Concept Application


1. Explain how Deep Packet Inspection (DPI) at the Network and Transport layers of the OSI model allows a startup to isolate specific traffic bottlenecks causing slow internet speeds despite high available bandwidth.

ANS:

Deep Packet Inspection (DPI) examines not only the packet header but also the packet data to identify the type of traffic passing through a network.

In a startup experiencing slow internet speeds despite having high bandwidth, DPI helps by:

- Analyzing packets at the Network Layer (Layer 3) to identify source IP, destination IP, and routing information.

- Inspecting Transport Layer (Layer 4) details such as TCP/UDP ports, sessions, retransmissions, and packet loss.

- Identifying which applications or users are consuming the most bandwidth (e.g., video streaming, large downloads, cloud backups).

- Detecting network issues like congestion, high latency, excessive retransmissions, or abnormal traffic.

- Helping administrators prioritize critical business traffic and limit unnecessary traffic to improve network performance.

Conclusion:
By using Deep Packet Inspection (DPI) at the Network (Layer 3) and Transport (Layer 4) of the OSI model, the startup can accurately identify the source of traffic bottlenecks, troubleshoot slow internet performance, and optimize network efficiency even when sufficient bandwidth is available.

2. Explain the distinct roles of DHCP and DNS in a scenario where startup devices are successfully assigned IP addresses but remain unable to resolve external web addresses for internet access.

ANS:

DHCP (Dynamic Host Configuration Protocol) and DNS (Domain Name System) have different roles in a network.

In this scenario:

- DHCP is working correctly because it successfully assigns IP addresses, subnet mask, default gateway, and DNS server information to the startup's devices.

- DNS is responsible for converting website names (such as www.google.com) into IP addresses that computers use to connect to websites.

- If devices receive an IP address but cannot open websites using their names, the problem is likely with the DNS server or its configuration.

- Users may still be able to access a website by entering its IP address directly, which confirms that internet connectivity exists but name resolution is failing.

Conclusion:
DHCP provides the network configuration needed for devices to join the network, while DNS translates domain names into IP addresses. In this scenario, DHCP is functioning properly, but a DNS issue prevents users from accessing websites by name.

3. Explain the necessity of Inter-VLAN Routing and the specific role of static routes when a startup requires communication between two isolated departments on the same physical switch.

ANS:

When a startup has two departments (for example, Sales and HR) on separate vlans, the devices are placed in different networks and cannot communicate directly, even though they are connected to the same physical switch.

- Inter-VLAN Routing is required to allow communication between these different vlans.

- A router or a Layer 3 switch performs the routing by forwarding data packets from one VLAN to another.

- Static routes are manually configured routes that tell the router the path to reach a specific network.

- They are useful in small startup networks because they are simple, secure, and do not require dynamic routing protocols.

Conclusion:
Inter-VLAN Routing enables communication between isolated vlans, while static routes define the path that data should take between networks, ensuring reliable communication between departments on the same physical switch.

4. Explain the performance differences between ICMP and HTTP/HTTPS protocols that would cause a website to experience high latency while a ping test returns a fast, successful response

ANS:

A ping uses the ICMP (Internet Control Message Protocol) to check whether a device is reachable and to measure basic network latency.

HTTP/HTTPS are application-layer protocols used to load web pages and transfer website data.

If a ping is fast but the website loads slowly, it means:

- ICMP only sends small test packets and checks network connectivity.

- HTTP/HTTPS involve additional processes such as DNS lookup, TCP connection, SSL/TLS handshake (for HTTPS), and downloading web content (images, scripts, etc.).

- The website may experience delays due to server overload, slow web application, large web pages, or network congestion, even though the network connection itself is healthy.

Conclusion:
A fast ICMP ping shows that the network path is working correctly, while HTTP/HTTPS performance depends on additional factors like DNS resolution, server response time, encryption, and webpage content. Therefore, a website can have high latency even when the ping response is fast.

5. Explain the conceptual logic of Firewall Access Control Lists (acls) and the strategic placement of a firewall to restrict unauthorized traffic between a startup’s management and general staff departments.

ANS:

A Firewall protects a network by monitoring and controlling incoming and outgoing traffic based on predefined security rules.

Access Control Lists (acls) are a set of rules that determine which traffic is allowed or blocked based on factors such as:

- Source IP address

- Destination IP address

- Protocol (TCP/UDP)

- Port number

In a startup with Management and General Staff departments:

- The firewall should be placed between the two department networks (vlans) or at the network gateway so that all traffic between them passes through it.

- Acls can be configured to:

  - Allow management staff to access shared resources or staff systems if required.

  - Block general staff from accessing management systems and sensitive data.

  - Permit only authorized services (such as email or file sharing) while denying all other traffic.

Conclusion:
Firewall acls control which network traffic is permitted or denied, and placing the firewall between the management and staff networks ensures that only authorized communication is allowed, improving the startup's network security.

Section B: Practical Application

Tools: Cisco Packet Tracer, Wireshark, CLI Terminal.

Task 1: Design a VLSM subnet scheme for 3 departments requiring 15 hosts each using $192.168.10.0/24$.

ANS:

Network: 172.16.20.0/24
host: 50

For 50 hosts, i need a /26 subnet mask.

| Vlan | Network address | Host range | Broadcast address |
| --- | --- | --- | --- |
| Vlan 10 | 172.16.20.0/26 | 172.16.20.1 – 172.16.20.62 | 172.16.20.63 |
| Vlan 20 | 172.16.20.64/26 | 172.16.20.65 – 172.16.20.126 | 172.16.20.127 |
| Vlan 30 | 172.16.20.128/26 | 172.16.20.129 – 172.16.20.190 | 172.16.20.191 |

Subnet mask: 255.255.255.192

Task 2: configure vlans 10, 20, and 30 on a cisco switch and assign physical interfaces to each.

ANS:

enable

configure terminal

vlan 10

name sales

vlan 20

name finance

vlan 30

name marketing

interface fa0/4

switchport mode access

switchport access vlan 10

interface fa0/5

switchport mode access

switchport access vlan 20

interface fa0/6

switchport mode access

switchport access vlan 30

Task 3: set up a dhcp pool on a router/l3 switch to provide dynamic addressing for the vlan 10 subnet.

ANS:

ip dhcp pool VLAN10

network 192.168.10.0 255.255.255.224

default-router 192.168.10.1

dns-server 192.168.10.1

Task 4: Capture a network trace in Wireshark and identify the IP addresses in the DNS query and HTTP GET headers.

ANS:

Yes, you can write it like this:

Dns query:

- Source ip = client ip address

- Destination ip = dns server ip address

- Query name = requested domain name (e.g., google.com)

Http get request:

- Source ip = client ip address

- Destination ip = web server ip address

- Method = get

- Host = requested website

- Can be viewed by applying the http or http.request filter in wireshark.

This format is simple, concise, and suitable for practical assignment submissions.

Section C: Integrated Case Study

Scenario: TechLaunch, a startup with 40 employees, is experiencing "flapping" network connectivity. Employees in the Marketing wing report that their internet drops for 30 seconds every few minutes. The Engineering wing, located on a different VLAN, remains perfectly stable. A ping test from a Marketing PC to the Core Switch shows a consistent 25% packet loss and fluctuating latency.

Questions:

1. Based on the symptoms, identify the two most likely OSI layers where this fault is occurring.

ANS:

 The issue is most likely at:

- Layer 1 (Physical Layer)

- Layer 2 (Data Link Layer)

2. Outline a 4-step "Bottom-Up" troubleshooting sequence to isolate this specific issues.

ANS:

Answer:

4-Step Bottom-Up Troubleshooting Sequence

- Check the Physical Layer

  - Verify the Ethernet cable is properly connected and the link LEDs are ON.

- Check the Data Link Layer

  - Ensure the switch port is enabled and assigned to the correct VLAN.

- Check the Network Layer

  - Verify the PC has the correct IP address, subnet mask, and default gateway using the ipconfig command.

- Test Connectivity

  - Use the ping command to test connectivity to the default gateway and then to the destination device to identify where communication fails.

3. List three CLI commands you would run on the switch to check for cable faults or broadcast storms.

ANS:

Answer:

Three CLI commands to check for cable faults or broadcast storms:

show interfaces

- Checks the status of switch interfaces and looks for errors such as CRC errors, collisions, or input/output errors.

show interfaces counters errors

- Displays error counters for each interface to identify cable faults or damaged ports.

show spanning-tree

- Verifies the Spanning Tree Protocol (STP) status and helps detect switching loops that can cause broadcast storms.

4. If the issue is a "Switching Loop," explain which protocol should have prevented this and why it might have failed.

ANS:

Spanning Tree Protocol (STP) is designed to prevent switching loops by blocking redundant paths. If a loop occurs, STP may be disabled or incorrectly configured.

5. Propose a long-term configuration change to ensure an error in one department doesn't impact the rest of the office.

ANS:

Answer:

Long-Term Configuration Change:

- Configure separate VLANs for each department to isolate network traffic.

- Enable Inter-VLAN routing through a router or Layer 3 switch for controlled communication.

- Enable Port Security and Storm Control on switch ports to prevent unauthorized devices and limit broadcast storms.

- Use Spanning Tree Protocol (STP) to prevent switching loops and improve network stability.